### ENVIRONMENTAL PROTECTION MEASURES

### General Environmental Protection Measures

In addition to the resource-specific environmental protection measures described below, the following general environmental protection measures shall apply to all project-related construction activities.

Environmental Protection Measure 1: Restrict Construction Activities to Disturbed Areas.

All construction activities conducted by CVIN and its contractors shall take place within the disturbed road right-of-way or in areas that have been surveyed by a qualified biologist and qualified cultural resource specialist and have been found to be devoid of sensitive resources. Staging and equipment lay-down areas shall be established only on disturbed areas and shall be located on private lands in existing contractor yards; existing commercial areas used for storing and maintaining equipment; previously cleared, graded, or paved areas; portions of the construction right-of-way; or level areas where grading and vegetation clearing are not required.

Environmental Protection Measure 2: Comply With All Applicable Laws, Ordinances, Regulations, and Policies.

Prior to project construction, CVIN shall consult with all appropriate jurisdictions to ensure consistency with applicable federal, state, regional, and local (city and/or county) regulations, laws, policies, and ordinances relevant to construction and operation of the Proposed Action. Construction and operation of the project shall comply with all applicable laws, regulations, policies, and ordinances.

### Resource-Specific Environmental Protection Measures

##### Noise

Environmental Protection Measure 4.2-1: Reduce Noise Levels from On-Site Construction Equipment.

CVIN and/or its construction contractor(s) shall implement the following environmental protection measures during construction to reduce temporary, short-term noise levels from on-site construction equipment:

- Construction equipment shall be properly maintained and equipped with noise control, such as mufflers, in accordance with manufacturers’ specifications.

- Construction activities shall be limited to the hours outlined in Table 3.2-1 in Section 3.2, “Noise;” or for those jurisdictions that do not currently have an adopted restriction governing construction operations, activities shall be limited to 7 a.m. to 7 p.m. Monday through Friday and 8 a.m. to 5 p.m. Saturday, during which such activities are exempt from noise levels identified in applicable standards. Emergency work to protect life or property is exempt from these hourly limits and applicable noise standards.

- If construction activities must run past exempted hours, any nearby sensitive receptors (located less than 1,300 feet from those activities) shall be given at least 48 hours notice of such activities by CVIN or its contractor(s). Before initiating construction activities during exempted hours, CVIN or its contractor(s) shall prepare a plan demonstrating how appropriate noise-reducing measures (such as erecting temporary sound barriers) would be implemented to maintain the applicable noise level standards. The plan shall be submitted to the governing agency of the area (such as the local county or city) for review and approval, and shall be implemented during all construction activities occurring outside of exempted hours.

- Construction equipment shall be arranged to minimize travel adjacent to noise-sensitive receptors and turned off during prolonged periods of nonuse.

- A disturbance coordinator shall be designated and the person’s telephone number shall be conspicuously posted around the project site and supplied to noise-sensitive receptors. The disturbance coordinator shall receive all public complaints and be responsible for determining the cause of the complaint and implementing any feasible measures to alleviate the problem.

- Construction equipment shall be staged and construction employee parking shall be located in designated areas only.

##### Air Quality

Environmental Protection Measure 4.3-1: Implement All Applicable Emissions Control Measures for Construction Equipment Operating within the San Joaquin Valley.

CVIN and its construction contractors shall implement all applicable emissions control measures for construction equipment, as required by law, whenever such equipment is operating within the San Joaquin Valley. The measures to be implemented shall include but not be limited to the following required control measures:

- Use alternative-fueled or catalyst-equipped diesel construction equipment.

- Minimize idling time (e.g., 10-minute maximum).

- Limit the hours of operation of heavy-duty equipment and/or the amount of equipment in use.

- Replace fossil-fueled equipment with electrically driven equivalents (provided they are not run via a portable generator set).

- Curtail construction during periods of high ambient pollutant concentrations; this may include ceasing construction activity during the peak hour of vehicular traffic on adjacent roadways.

- Implement activity management (e.g., rescheduling activities to reduce temporary and short-term effects). No more than one crew of each type of construction activity (trenching, drilling, fiber blowing, etc.) shall occur at the same time on a particular construction segment.

- Use the newest equipment available to try and maintain a Tier 1 fleet equipment average.

- Maintain records of any equipment operating within the San Joaquin Valley APCD. These records, to be maintained by the construction contractors, shall include equipment number and weekly hours of operation within the district. At the completion of Proposed Action construction, the total annual emissions for the project shall be calculated by or to the satisfaction of the San Joaquin Valley APCD and CVIN shall pay the applicable air quality mitigation fees.

Environmental Protection Measure 4.3-2: Implement All Applicable Dust Control Measures, as Required by the San Joaquin Valley Air Pollution Control District.

CVIN and its construction contractors shall comply with the San Joaquin Valley APCD’s Regulation VIII, “Fugitive Dust PM10 Prohibitions,” and implement all applicable control measures, as required by law. Regulation VIII contains but is not limited to the following required control measures:

- Prewater site sufficient to limit visible dust emissions (VDE) to 20% opacity.

- Phase work to reduce the amount of disturbed surface area at any one time.

- During active operations, apply water or chemical/organic stabilizers/suppressants sufficient to limit VDE to 20% opacity.

- During active operations, construct and maintain wind barriers sufficient to limit VDE to 20% opacity.

- During active operations, apply water or chemical/organic stabilizers/suppressants to unpaved haul/access roads and unpaved vehicle/equipment traffic areas sufficient to limit VDE to 20% opacity and meet the conditions of a stabilized unpaved road surface.

- When handling bulk materials, apply water or chemical/organic stabilizers/suppressants sufficient to limit VDE to 20% opacity.

- When handling bulk material, construct and maintain wind barriers sufficient to limit VDE to 20% opacity and with less than 50% porosity.

- When storing bulk materials, comply with the conditions for a stabilized surface as listed above.

- When storing bulk materials, cover bulk materials stored outdoors with tarps, plastic, or other suitable material and anchor in such a manner that prevents the cover from being removed by wind action.

- When storing bulk materials, construct and maintain wind barriers sufficient to limit VDE to 20% opacity and with less than 50% porosity. If using fences or wind barriers, apply water or chemical/organic stabilizers/suppressants to limit VDE to 20% opacity or use a three-sided structure with a height at least equal to the height of the storage pile and with less than 50% porosity.

- Load all haul trucks such that the freeboard is not less than 6 inches when material is transported across any paved public access road sufficient to limit VDE to 20% opacity.

- Apply water to the top of the load sufficient to limit VDE to 20% opacity.

- Cover haul trucks with a tarp or other suitable cover.

- Clean the interior of the cargo compartment or cover the cargo compartment before the empty truck leaves the site.

- Prevent carryout and trackout, or immediately remove carryout and trackout when it extends 50 feet or more from the nearest unpaved surface exit point of a site.

- Clean up carryout and trackout by either manually sweeping and picking it up; operating a rotary brush or broom accompanied or preceded by sufficient wetting to limit VDE to 20% opacity; operating a PM10-efficient street sweeper that has a pickup efficiency of at least 80%; or flushing with water, if curbs or gutters are not present, and where the use of water would not result as a source of trackout material or result in adverse effects on stormwater drainage systems or violate any National Pollutant Discharge Elimination System (NPDES) permit program.

Environmental Protection Measure 4.3-3: Reduce Emissions of Naturally Occurring Asbestos Dust.

CVIN and/or its construction contractor(s) shall implement the following measures during construction within one-half mile of state-identified naturally occurring asbestos (NOA) areas:

- It is assumed that any segment crossing within one-half mile of a state-identified NOA area is operating in serpentine, or ultramafic rock. Therefore, CVIN and/or its contractor(s) shall comply with all requirements outlined in the California Air Resources Board (ARB) Asbestos Air Toxic Control Measures for Construction, Grading, Quarrying, and Surface Mining Operations to reduce potential effects from exposure to NOA to the maximum extent feasible. These requirements shall include:

- preparation and implementation of an asbestos dust mitigation plan, which must be approved by the local air district before construction begins, and must be implemented at the beginning of construction and grading activities and maintained throughout such activities; and

- implementation of an asbestos health and safety program (as required under Title 8, Section 1529, “Asbestos,” of the California Code of Regulations (CCR) [i.e., 8 CCR Section 1529]).

- The asbestos dust mitigation plan, per 17 CCR Sections 93105(e) (2) and 93105(e) (4), shall specify dust mitigation practices that are sufficient to ensure that no equipment or operation emits dust that is visible crossing property lines. The plan also shall include track-out prevention and control measures, control measures for disturbed surface area and storage piles that would remain inactive for more than 7 days, postconstruction stabilization, and asbestos monitoring, if required. Examples of these may include but shall not be limited to surface wetting, surface covering, surface crusting, application of chemical dust suppressants or stabilizers, installation of wind barriers, construction area speed limits, truck spillage controls, and establishment of vegetative covers. In addition, the asbestos dust mitigation plan shall include recordkeeping and reporting requirements that document the results of any air monitoring, geologic evaluation, and asbestos bulk sampling.

The asbestos health and safety program shall be implemented if permissible exposure limits for airborne asbestos are found to be exceeded within the project area. Implementation shall include applicable construction employee protection measures as defined under 8 CCR Section 1529(g) and any additional measures required by the California Occupational Safety and Health Administration to reduce exposure of construction employees to airborne asbestos.

Environmental Protection Measure 4.3-4: Prohibit Open Burning of Vegetative Waste within the Northern Sierra Air Quality Management District.

CVIN and/or its construction contractor(s) shall prohibit open burning of vegetative waste within the Northern Sierra Air Quality Management District

Environmental Protection Measure 4.3-5: Reduce Greenhouse Gas Emissions from Construction Equipment.

CVIN and its construction contractors shall implement the following control measures, as required by law:

- Improve fuel efficiency from construction equipment:

- Minimize idling time by either shutting equipment off when not in use or reducing idling time to no more than 3 minutes. (A 5-minute limit is required by the state airborne toxics control measure [13 CCR Sections 2449(d) (3) and 2485].) Post clear signage at site entrances that explains this requirement to workers.

- Maintain all construction equipment in proper working condition according to manufacturer’s specifications. The equipment must be checked by a certified mechanic and determined to be running in proper condition before it is operated.

- Train equipment operators in proper use of equipment.

- Use properly sized equipment for tasks.

- Use equipment with new technologies (repowered engines, electric drive trains).

- Use trucks equipped with on-road engines for on-site material hauling (if determined to be less emissive than the off-road engines).

- Use alternative fuels such as propane or solar, or electrical power, for generators at construction sites.

- Use an ARB-approved low-carbon fuel for construction equipment. (Oxides of nitrogen [NOX] emissions from the use of low-carbon fuel must be reviewed and increases mitigated.)

- Minimize the amount of concrete for paved surfaces or use a low-carbon concrete option.

- Use SmartWay certified trucks for deliveries and equipment transport.

- Develop a plan to efficiently use water for adequate dust control.

##### Water Resources

Environmental Protection Measure 4.5-1: Prepare and Implement Storm Water Pollution Prevention Plans.

Storm water pollution prevention plans (SWPPPs) shall be prepared and implemented to protect water quality. The SWPPPs shall identify and specify the activities that might cause discharge of construction-related contaminants, including sediment, to major and localized receiving waters (e.g., culverts, ditches, swales) during storms, and the Best Management Practices (BMPs) that would be employed to control pollutant discharges. The SWPPPs shall include a sediment and erosion control plan that complies with county and city grading and excavation requirements. The plan’s BMPs shall include measures to control erosion, stream sedimentation, dust, and soil mass movement, and design provisions for stormwater management systems to prevent the degradation of water quality related to stormwater runoff and soil erosion from the Proposed Action and minimize increases in turbidity in receiving waters.

Best Management Practices (BMP) designations shall be based on those used by the California Stormwater Quality Association handbook of construction BMPs (CASQA 2009). BMPs may include but shall not be limited to:

- Scheduling—Proper scheduling shall be used to assist in identifying ways to minimize disturbed areas, thus allowing for a reduction in the active project area requiring protection and minimizing the length of time that disturbed soils are exposed to erosive processes.

- Preservation of Existing Vegetation—Existing vegetation shall be preserved to the maximum extent practicable to facilitate protection of surfaces from erosion and provide sediment control benefits. Sensitive areas such as those defined in Section 3.4, “Biological Resources,” shall also be clearly identified on construction drawings and protected.

- Hydraulic Mulch, Straw Mulch, and Wood Mulching—Various mulches shall be used for temporary soil stabilization on surfaces with little or no slope.

- Geotextiles, Plastic Covers, and Erosion Control Blankets/Mats—Geotextiles, plastic covers, and erosion control blankets/mats shall be used for erosion control on flat or, usually, sloped surfaces, channels, and stockpiles.

- Silt Fence—A silt fence consisting of fabric shall be used as a temporary sediment barrier to retain sediment from small disturbed areas by reducing the velocity of sheet flows.

- Gravel Bag Berm and Sand/Gravel Bag Barrier—A gravel bag berm and sand/gravel bag barrier (gravel-filled fabric bags) shall be used as a temporary sediment barrier to retain sediment from small disturbed areas by reducing the velocity of sheet flows.

- Storm Drain Inlet Protection—Inlet protection devices shall be used to detain or filter sediment-laden runoff to facilitate the removal of sediment from waters before discharge.

- Stabilized Construction Entrance/Exit—A graveled area or pad shall be built at any point where vehicles enter and leave a construction site. This BMP shall provide a buffer area where vehicles can drop their mud and sediment to avoid transporting it onto public roads, to control erosion from surface runoff, and to help control dust.

- Runoff Control Measures—Runoff control measures shall be implemented as necessary during construction. Measures may include graded surfaces to redirect sheet flow, diversion dikes, or berms that force sheet flow around a protected area, and stormwater conveyances (swales, channels, gutters, drains, sewers) that intercept, collect, and redirect runoff. Diversions can be either temporary or permanent. Construction of temporary diversions involves excavating a channel and placing the spoil in a dike on the downgradient side of the channel, and placing gravel in a ridge below an excavated swale. Permanent diversions, constructed of more permanent materials, may be used to divide a site into specific drainage areas, sized to capture and carry flows from a storm event of a specific magnitude. A water bar is a specific kind of runoff diversion that is constructed diagonally at intervals across a linear sloping surface such as a road or right-of-way that is subject to erosion. Water bars are meant to interrupt accumulation of erosive volumes of water through their periodic placement down the slope, and divert the resulting segments of flow into adjacent undisturbed areas for dissipation.

- Groundwater Extracted during Construction—Dewatering shall be discharged to land if feasible. If land disposal is not feasible, the construction contractor(s) shall obtain an NPDES permit for construction dewatering activity and comply with all associated requirements, including maximum sediment loads and prohibition of toxic substance discharges.

- Personnel Training Requirements and Procedures—Construction personnel shall be trained to ensure that workers are aware of permit requirements and proper installation methods for BMPs specified in the SWPPPs.

- Appropriate Personnel—The individuals responsible for supervisory duties related to SWPPP implementation and BMP maintenance shall be specifically designated by CVIN and/or its construction contractors.

All construction contractors shall retain a copy of the approved SWPPPs on the construction site. Clearing and grading shall be limited to the minimum necessary to complete the Proposed Action and shall be confined to the established rights-of-way of roads and state highways. Boundaries of clearing shall be clearly marked. Under the sediment and erosion control plan, the project area shall be stabilized when construction is complete, and postconstruction BMPs and monitoring shall be implemented to ensure that sediment from disturbed areas would not mobilize. Before allowing construction to begin, CVIN shall verify that a notice of intent and SWPPPs have been filed. CVIN or its designated agent shall routinely inspect the active project site to verify that the BMPs specified in the SWPPP were properly implemented and maintained. Inspection reports shall be included in project files. Project staff and contractors shall immediately stop any activities that resulted in noncompliance and shall implement appropriate corrective actions.

Environmental Protection Measure 4.5-2: Prepare and Implement an Inadvertent Release (Frac-out) Contingency Plan for Directional Drilling (Boring) under Sensitive Drainages and Waterways.

At all streams that provide important habitat, contribute to water quality, or support sensitive or listed aquatic species, the conduit shall be installed by either conducting directional drilling under the drainage or attaching the conduit to a bridge crossing the drainage.

CVIN shall prepare and implement an inadvertent release (frac-out) contingency plan that describes the procedures to be used and equipment and supplies that shall be available on-site during directional drilling at drainages or other sensitive wetlands. The primary purpose of the inadvertent release (frac-out) contingency plan is to identify methods and procedures to be used to prevent, limit, and contain seepage of bentonite (a fine, nontoxic clay used to lubricate the bore during directional drilling) or other similar material into waters or wetlands. Each construction contractor undertaking directional drilling under streams and wetlands shall become familiar with and implement the inadvertent release (frac-out) contingency plan.

The inadvertent release (frac-out) contingency plan shall be prepared and implemented before any directional drilling occurs. The plan shall include measures to:

- minimize the potential for a frac-out;

- provide for the timely detection of frac-outs;

- protect areas that are considered environmentally sensitive (streams, wetlands, other biological resources, cultural resources); and

- ensure an organized, timely, and “minimum-effect” response in the event of a frac-out and release of drilling mud.

The plan shall require that a geotechnical engineer or qualified geologist make recommendations regarding the suitability of the formations to be bored to minimize the potential for frac-out conditions. The plan’s BMPs will be determined by the geotechnical engineer or geologist based on site-specific hydrogeologic characteristics and may include, but not be limited to, the measures identified below.

- A full-time monitor shall be present at the construction site during all drilling activities to look for signs of frac-out conditions or lowered pressure readings on drilling equipment. If a frac-out is identified, all work shall stop, including the recycling of drilling lubricant.

- The frac-out shall be monitored for 4 hours following identification or as specified by the geotechnical engineer or geologist to determine if the drilling mud congeals and seals the frac-out location. If this occurs no other action shall be taken that would potentially suspend the drilling mud in the water column.

- If drilling mud does not congeal, isolation/containment barriers to be determined by the geotechnical engineer or geologist, such as underwater boom and curtains, shall be implemented.

- If determined necessary by the monitor, a spill response team shall be called in to contain and clean up excess drilling mud in the water. Phone numbers of spill response teams in the area will be available on-site.

##### Biological Resources

Environmental Protection Measure 4.6-1: Restrict Construction Activities to Disturbed Areas That Do Not Support Sensitive Biological Resources.

All cable installation shall occur within the disturbed road right-of-way. All construction activities associated with cable installation shall also take place within the disturbed road right-of-way or in areas that have been surveyed by a biologist approved by the U.S. Fish and Wildlife Service (USFWS) and California Department of Fish and Game (DFG) and have been found to be devoid of sensitive resources. Staging and equipment lay-down areas shall be established only on disturbed areas and shall be located on private lands in existing contractor yards; existing commercial areas used for storing and maintaining equipment; previously cleared, graded, or paved areas; portions of the construction right-of-way; or level areas where grading and vegetation clearing are not required. To ensure that sensitive environmental resources are adequately protected or are avoided, the locations of staging areas shall be established in consultation with an agency-approved biologist and shall be subject to all environmental protection measures identified herein.

All project activity shall occur as close as is practical to the edge of the paved road surface, or within the paved road where necessary, to avoid impacts on sensitive biological resources. No project activity shall be allowed outside of the graded and routinely maintained road right-of-way unless the area has been confirmed by an agency-approved biologist as devoid of any sensitive biological resources and the potential for direct and indirect impacts on sensitive biological resources has been determined to be negligible.

Environmental Protection Measure 4.6-2: Identify Environmentally Sensitive Areas on Construction Plans and Specifications.

Construction plans and specifications for each segment of the fiber optic route shall identify the location of sensitive biological resources as environmentally sensitive areas (ESAs). The ESA shall be delineated by an agency-approved biologist prior to the initiation of ground-disturbing activities. Temporary construction fencing shall be installed around ESAs located within the road right-of-way. If an agency-approved biologist has determined that indirect construction impacts could occur to sensitive biological resources near but not within the right-of-way, ESA fencing shall be established to protect the area deemed necessary by the agency-approved biologist to avoid such indirect impacts. Fencing shall remain in place for the duration of construction within each segment. No vehicles, heavy equipment, vegetation removal, storage of material, or surface-disturbing activities or other construction shall be permitted within the ESAs.

Environmental Protection Measure 4.6-3: Biological Monitors Shall Supervise All Construction Activities within 250 Feet of Sensitive Biological Resources.

An agency-approved biologist shall be present during any construction activities occurring within 250 feet of sensitive biological resources. The biologist shall monitor site mobilization and construction related ground disturbance such as plowing, trenching, or directional drilling, and shall work with CVIN’s construction/operation manager to ensure compliance with all environmental protection measures. The agency-approved biologist shall maintain a log of daily monitoring notes that can be summarized and transmitted to the USFWS and DFG upon request.

Environmental Protection Measure 4.6-4: Develop and Implement a Worker Environmental Awareness Program for Protection of Biological Resources.

Prior to construction, CVIN shall develop and implement a Worker Environmental Awareness Program (WEAP) to educate workers about sensitive biological resources occurring in and near the project area. All field staff, including employees, contractors, and subcontractors who work on the project site during construction, shall be required to participate in the program. The WEAP shall contain specific information related to the each segment of the route and shall inform workers about the locations and types of ESAs potentially occurring in or near the route and about the environmental protection measures that must be implemented to avoid impacts on the sensitive biological resources within the ESAs.

The WEAP shall provide information on the distribution of listed and other special-status species, the general behavior and ecology of these species, their sensitivity to human activities, their legal protection, the penalties for violation of state and federal laws, reporting requirements, and measures to implement in the event that a species is found during construction. A fact sheet with all this information shall be prepared and distributed to all field staff. All employees, contractors, and subcontractors shall receive training on the WEAP prior to working on the site. Employees shall sign a form stating that they attended the training and that they will comply with all environmental protection measures. Forms shall be filed with CVIN’s construction/operation manager and made available to the USFWS and DFG upon request.

Environmental Protection Measure 4.6-5: Implement Best Management Practices to Protect Water Quality, Aquatic Habitat, and Special-Status Species.

The measures described below shall be implemented by CVIN before and during construction.

- No Loss of Aquatic Habitat. The project shall be constructed to achieve no loss of area, functions, or values of aquatic habitat, including wetlands and waters of the state and United States.

- No Construction Activities in Water. No plowing, trenching, grading, or other construction activities shall take place in surface water that is hydrologically connected to a waterway.

- No Vegetation Removal from Waterways. No aquatic, emergent, or riparian vegetation shall be disturbed or removed from waterways within the project sites.

- Setback from Drainages. Bore pit excavations shall be located at least 20 feet from boundary of waters of the state or of the United States. DFG may establish a greater setback at certain drainages if site conditions warrant, as described in any Streambed Alteration Agreement issued for this project. A lesser setback may be approved by the agency-approved biologist if appropriate precautions are in place to prevent water quality impacts only after securing concurrence from DFG.

- Hazardous Spill Prevention. All vehicles and equipment shall be maintained in proper working condition to minimize the potential for fugitive emissions of motor oil, antifreeze, hydraulic fluid, grease, or other hazardous materials. The agency-approved biologist shall be informed of any hazardous spills immediately as directed in the Spill Prevention Plan (Environmental Protection Measure 4.12-1) prepared for this project. Hazardous spills shall be cleaned up immediately and the contaminated soil properly disposed of at a licensed facility. Servicing of construction equipment shall take place only within designated areas. Service/maintenance vehicles shall carry a bucket and pads to absorb leaks or spills. All staging areas shall be located at least 150 feet away from streams, drainages, and wetlands. Equipment washing shall occur only in locations where water cannot flow into a drainage channel.

- Implement Erosion Control. Standard erosion control measures shall be implemented for all phases of construction where sediment runoff from exposed slopes has the potential to enter waterways. Sediment and other flow-restricting materials shall be moved to a location where they cannot be washed back into the stream. All disturbed soils and roads within the project site shall be stabilized to reduce erosion potential, both during and following construction. Areas of disturbed soils (access and staging areas) sloping toward drainages shall be stabilized to reduce erosion potential. Erosion control measures shall avoid the introduction and spread of invasive weeds, and straw bales used for sediment barriers or shall be certified as weed-free.

- Maintain Original Topography and Hydrology. Upon completion of construction, all work areas shall be restored to preexisting contours and conditions so that flow lines and preproject hydrology and topography remain unchanged.

- Implement Dust Control Measures. Unpaved areas subject to disturbance from construction activities shall be watered as needed to control fugitive dust in habitats adjacent to the right-of-way.

- Prepare Water Quality Protection Plans: Prior to construction CVIN shall prepare and implement the following plans to avoid and minimize potential water quality impacts:

- Spill Prevention Plan (Environmental Protection Measure 4.12-1),

- Storm Water Pollution Prevention Plans (Environmental Protection Measure 4.5-1), and

- Inadvertent Release (Frac-out) Contingency Plan for Directional Drilling (Environmental Protection Measure 4.5-2).

Environmental Protection Measure 4.6-6: Avoid Impacts to Birds Protected under the Migratory Bird Treaty Act.

Prior to commencement of construction activities between February 1 and September 15, an agency-approved biologist shall conduct nest surveys within all portions of the right-of-way that could potentially support nesting birds protected under the Migratory Bird Treaty Act. Surveys shall be conducted no more than 10 days prior to the initiation of construction, and shall be repeated if a lapse in construction activity of more than 10 days occurs. If the agency-approved biologist determines that any active nests located within or near the right-of-way could be disturbed by construction, avoidance measures shall be implemented to protect the nest, eggs, and young. These measures may include establishment of non-disturbance buffer around the nest until the young have fledged, and/or shifting construction activities to another portion of the alignment until nesting is completed. Construction activities may not resume in the vicinity of nesting birds until the agency-approved biologist has verified that such activities would not result in disturbance to nesting birds.

Environmental Protection Measure 4.6-7: Avoid Impacts to Burrowing Owls.

An agency-approved biologist shall conduct surveys for burrowing owls within the right-of-way in areas which could potentially support this species, including portions of the alignment adjacent to agricultural lands, grassland, or ruderal habitat. Survey protocol shall be consistent with that described in the California Burrowing Owl Consortium guidelines, Burrowing Owl Survey Protocol and Mitigation Guidelines (CBOC 1993). If burrowing owls are detected within or near the right-of-way and if, in the opinion of the agency-approved biologist, could be disturbed by construction activities, avoidance measures shall be implemented as recommended in the DFG’s Staff Report on Burrowing Owl Mitigation (DFG 1995). If implementation of those recommended measures is infeasible due to physical constraints or private property considerations, DFG shall be consulted prior to construction activities to establish an appropriate non-disturbance buffer or other appropriate avoidance measures.

Environmental Protection Measure 4.6-8: Avoid Impacts to Swainson’s Hawks.

Prior to commencement of construction activities between February 1 and September 15 in areas that could support nesting Swainson’s hawks, an agency-approved biologist shall conduct surveys for this species. The surveys shall be conducted within and near the right-of-way, and shall include all areas which could provide nesting habitat for Swainson’s hawks and are within visual or auditory range of project construction activities. Survey protocol shall be consistent with that described by the Swainson’s Hawk Technical Advisory Committee (SWHA TAC 2000). If nesting Swainson’s hawks are detected within or near the right-of-way and if, in the opinion of the agency-approved biologist, could be disturbed by construction activities, DFG shall be consulted prior to construction to establish an appropriate non-disturbance buffer.

Environmental Protection Measure 4.6-9: Avoid Indirect Impacts to Vernal Pool Species.

Water quality protection measures shall be implemented to avoid indirect impacts on listed vernal pool species (Colusa grass, San Joaquin Valley Orcutt grass, hairy Orcutt grass, Greene’s tuctoria, vernal pool fairy and tadpole shrimp) potentially occurring in aquatic habitat within 250 feet of the project alignment. These shall include all appropriate environmental protection measures listed above, such as requirements to avoid any changes to the hydrology or topography of drainages, dust and erosion control measures, and implementation of water quality protection plans. In addition, silt fencing shall be installed between the work area and any aquatic habitat that could support listed vernal pool species occurring within 250 feet of the right-of-way. Silt fencing shall be installed before commencement of project-related soil-disturbance activities and shall remain in place until construction is completed within each segment. The agency-approved biologist shall inspect the placement of silt fencing and any other water quality protection measures prior to initiating ground-disturbing activities within 250 feet of habitat that could support listed vernal pool species.

Environmental Protection Measure 4.6-10: Avoid Impacts to Valley Elderberry Longhorn Beetle.

The measures described below shall be implemented by CVIN before and during construction to avoid impacts to valley elderberry longhorn beetle (VELB).

- VELB-1 - Maintain Buffer around Elderberry Shrubs. Where feasible, CVIN shall maintain a 100-foot buffer around elderberry shrubs (those with stems larger than 1-inch diameter at ground height) occurring in the action area. Where maintaining a 100-foot buffer zone is infeasible due to physical constraints and private property considerations, a minimum 20-foot buffer shall be maintained between the dripline of elderberry shrubs and the work area. Signs and ESA fencing shall be erected around elderberry shrubs buffer zones to keep construction away from the protected area.

- VELB-2 Include VELB Information in WEAP. As part of the WEAP, an agency-approved biologist shall provide information to all construction personnel about how to avoid impacts on VELB and elderberry shrubs and the possible consequences of non-compliance.

Environmental Protection Measure 4.6-11: Avoid Impacts to Giant Garter Snake.

The measures described below shall be implemented by CVIN before and during construction to avoid impacts to giant garter snake (GGS).

- GGS-1 No Construction in GGS Aquatic Habitat. No plowing, trenching, or other construction activities shall occur within giant garter snake aquatic habitat (marshes, sloughs, ponds, small lakes, low gradient streams, irrigation and drainage canals, and rice fields) in Colusa, Fresno, Merced, San Joaquin, Stanislaus, Sutter, and Yuba Counties. All giant garter snake aquatic habitat occurring within the right-of-way shall be flagged and designated as an ESA by an agency-approved biologist. Surface-disturbance activities shall not begin until the ESAs are delineated on the ground with temporary construction fencing, and the fencing shall remain in place for the duration of construction within each segment. No vehicles, heavy equipment, vegetation removal, storage of material, or surface-disturbance activities or other construction shall be permitted within the ESAs.

- GGS-2 Maintain Habitat Buffer. Where feasible, CVIN shall avoid all construction activities within 200 feet from the banks of giant garter snake aquatic habitat (marshes, sloughs, ponds, small lakes, low gradient streams, irrigation and drainage canals, and rice fields) in Colusa, Fresno, Merced, San Joaquin, Stanislaus, and Sutter Counties.

- GGS-3 Conduct Preconstruction Surveys. If plowing, trenching or other construction activities are unavoidable within 200 feet of giant garter snake aquatic habitat, the project area shall be surveyed by an agency-approved biologist no more than 24 hours prior to construction. The survey shall be repeated if a lapse in construction activity of two weeks or greater has occurred since the last survey. If a snake is encountered during construction, activities shall cease until appropriate corrective measures have been completed and it has been determined resumption of construction would not result in harm to the snake. Any sightings of GGS and any incidental take shall be reported to the USFWS immediately by telephone at (916) 414-6600 and to DFG. The biologist must submit all sightings to DFG Natural Diversity Data Base (NDDB) using a California Native Species Field Survey Form and provide copies to DFG and the USFWS.

- GGS-4 Limit Construction to Active Season. Construction activities within 200 feet of giant garter snake aquatic habitat in Colusa, Fresno, Merced, San Joaquin, Stanislaus, and Sutter Counties shall occur during the active season for giant garter snakes (May 1 and October 1) so that snakes can move and avoid danger. For construction activities proposed between October 2 and April 30, the USFWS Sacramento office shall be contacted prior to construction to determine if additional measures are necessary to minimize and avoid take.

- GGS-5. Include GGS Information in WEAP. The Worker Environmental Awareness Program shall include training that instructs workers to recognize giant garter snakes and their habitat(s), general behavior and ecology of this species, their sensitivity to human activities, their legal protection, penalties for violation of state and federal laws, reporting requirements, and measures to implement in the event that a giant garter snake is found during construction.

Environmental Protection Measure 4.6-12: Avoid Impacts to Blunt-nosed Leopard Lizard.

The measures described below shall be implemented by CVIN before and during construction to avoid impacts to blunt-nosed leopard lizard (BNLL).

- BNLL-1 Avoid Impacts to Blunt-nosed Leopard Lizard Habitat. In Madera, Fresno, Kings, Tulare, and Kern Counties where the blunt-nosed leopard lizard has potential to occur in habitat adjacent to the right-of-way, all construction activities shall be confined to the paved roadway or an area within 2 feet from the edge of pavement in areas where suitable habitat is present. Staging areas for equipment storage and vehicle parking or other work areas may be established within the right-of-way at a distance greater than 2 feet only after an agency-approved biologist has verified that no habitat for blunt-nosed leopard lizard would be affected by work at that location. An agency-approved biologist shall be present during all construction activities that occur within blunt-nosed leopard lizard habitat.

- BNLL-2. Avoid Impacts to Blunt-nosed Leopard Lizards Encountered During Construction. Any blunt-nosed leopard lizard encountered during construction shall be allowed to leave the work area unobstructed on its own.

- BNLL-3. Include Blunt-nosed Leopard Lizard Information in WEAP. The WEAP shall include training that instructs workers to recognize blunt-nosed leopard lizard habitat, general behavior and ecology of this species, including their frequent use of dirt roads for foraging and other activities, their sensitivity to human activities, their legal protection, penalties for violation of state and federal laws, reporting requirements, and measures to implement in the event that a blunt-nosed leopard lizard is found during construction.

Environmental Protection Measure 4.6-13: Avoid Impacts to Tipton Kangaroo Rat.

The measures described below shall be implemented by CVIN before and during construction to avoid impacts to Tipton kangaroo rat (TKR).

- TKR-1 Avoid Impacts to Tipton Kangaroo Rat Habitat. In Tulare County where Tipton kangaroo rat has potential to occur in habitat adjacent to the right-of-way, all construction activities shall be confined to the paved roadway or an area within 2 feet from the edge of pavement. Staging areas for equipment storage and vehicle parking or other work areas may be established within the right-of-way at a distance greater than 2 feet only after an agency-approved biologist has verified that no habitat for Tipton kangaroo rat would be affected by work at that location.

- TKR-2. Include Tipton Kangaroo Rat Information in WEAP. The WEAP shall include training that instructs workers to recognize Tipton kangaroo rat sign, their habitat(s), general behavior and ecology of this species, their sensitivity to human activities, their legal protection, penalties for violation of state and federal laws, reporting requirements, and measures to implement in the event that Tipton kangaroo rat is found during construction.

Environmental Protection Measure 4.6-14: Avoid Impacts to San Joaquin Kit Fox.

The measures described below shall be implemented by CVIN before and during construction to avoid impacts to San Joaquin kit fox (SJKF).

- SJKF-1. Conduct Preconstruction Surveys. No less than 14 days and no more than 30 days prior to the beginning of ground disturbance and/or construction activities within Kern, Tulare, Kings Fresno, Madera, and Merced counties, kit fox surveys shall be conducted by an agency-approved biologist within the right-of-way. Surveys shall identify any kit fox habitat features (including potential, atypical and known dens) on the project site and evaluate use by kit fox in accordance with guidelines described in Standardized Recommendations for Protection of the Endangered San Joaquin Kit Fox Prior to or During Ground Disturbance (USFWS 2011). Any potential kit fox dens detected in or near the right-of-way must be avoided during construction by establishment of exclusion zones, as described in the USFWS guidelines (USFWS 2011).

- SJKF-2. Cover Trenches and Inspect Pipes. To prevent inadvertent entrapment of San Joaquin kit fox and other animals, all excavated, steep-walled holes or trenches more than 2-feet deep shall be covered at the close of each working day by plywood or similar materials. If the trenches cannot be closed, one or more escape ramps constructed of earthen-fill or wooden planks shall be installed. Before such holes or trenches are filled, they shall be thoroughly inspected for trapped animals. All construction pipes, culverts, or similar structures with a diameter of 4-inches or greater that are stored at a construction site for one or more overnight periods shall be thoroughly inspected for kit foxes and other wildlife before the pipe is subsequently buried, capped, or otherwise used or moved in any way.

- SJKF-3. No Trash, Firearms, Pets on Project Site. All food-related trash items such as wrappers, cans, bottles, and food scraps should be disposed of in securely closed containers and removed at least once a week from the construction site. No firearms shall be allowed on the construction site, nor shall pets, such as dogs or cats, be permitted.

- SJKF-4. Include SKJK Information in Worker Environmental Awareness Program. The WEAP shall include training that instructs workers to recognize San Joaquin kit fox habitat, general behavior and ecology of this species, their sensitivity to human activities, their potential to occur in urban environments, restrictions on bring firearms or pets to the project site, or leaving trash, legal protection for kit fox, penalties for violation of state and federal laws, and reporting requirements.

Environmental Protection Measure 4.6-15: Avoid Impacts to California Tiger Salamander.

The measures described below shall be implemented by CVIN before and during construction to avoid impacts to California tiger salamander.

- CTS-1 Avoid Impacts to Tiger Salamander Upland Habitat. In Stanislaus, San Joaquin, Fresno, and Madera Counties where California tiger salamander has potential to occur in habitat adjacent to the right-of-way, all construction activities shall be confined to the paved roadway or an area within 2 feet from the edge of pavement where suitable upland habitat (i.e., rodent burrows) occurs. Work shall be conducted from the paved roadway with no plowing or trenching occurring more than 2 feet from the edge of pavement. Staging areas for equipment storage and vehicle parking or other work areas may be established within the right-of-way at a distance greater than 2 feet only after an agency-approved biologist has verified that no rodent burrows or other potential tiger salamander upland or aquatic habitat would be affected by work at that location.

- CTS-2 Include CTS Information in WEAP. The WEAP shall include training that instructs workers to recognize California tiger salamander upland habitat and refugia as well as aquatic habitat, general behavior and ecology of this species, their sensitivity to human activities, their legal protection, penalties for violation of state and federal laws, and reporting requirements.

Environmental Protection Measure 4.6-16: Implement Best Management Practices to Avoid the Spread of Noxious Weeds.

The measures described below shall be implemented by CVIN during construction to avoid the spread of noxious weeds.

- Project contractors shall ensure that any clothing, footwear, and equipment used during construction is free of soil, seeds, vegetative matter, or other debris or seed-bearing material before entering the construction area.

- Where it is not possible to keep equipment out of sites infested with noxious weeds, the equipment will be cleaned so that it is free of soil, seeds, vegetative matter, or other debris prior to being moved from infested sites to uninfested sites and prior to being transported out of the project area.

- CVIN shall ensure that any fill soil, mulch, seeds, and straw materials used during construction and for the implementation of best management practices are weed-free. Certified weed-free material will be used if available.

- All earthmoving equipment, gravel, fill, or other materials are required to be weed free. Sand, gravel, rock, or organic matter from an approved on-site source will be used when possible.

- The amount of ground and vegetation disturbance in construction areas will be minimized. All conduit installation shall occur within disturbed road rights-of-way where vegetation maintenance is an existing activity.

##### Table 2-3 provides a summary of the species-specific biological resources environmental protection measures that will be implemented within each segment of the project route.

##### Historic and Cultural Resources

Environmental Protection Measure 4.7-1: Implement Specific Measures for Full Avoidance of Adverse Effects on Each Cultural Resource Potentially Eligible for Listing in the National Register of Historic Places or the California Register of Historical Resources.

Specific avoidance methods that were developed by a qualified archaeologist who meets the requirements of the Secretary of the Interior’s Standards for Archaeology and Historic Preservation and which are identified in the cultural resources inventory report for the Proposed Action shall be implemented during project final design and during construction. The identified specific measures shall result in full avoidance of adverse impacts on each cultural resource that is potentially eligible for listing in the National Register of Historic Places (NRHP) or the CRHR. Identified measures include delineation of a specific rerouted alignment that avoids surface and subsurface manifestations of cultural resource sites; identification of specific sides of roads or other specified installation locations, and identification of specific lengths and depths for directional drilling (boring) underneath cultural resource sites. In situations where a historical bridge cannot be avoided by use of directional drilling below the bridge, the conduit shall be attached to the underside of the bridge, provided that the conduit is not visible from the top or sides of the bridge. The avoidance strategies shall be reviewed and approved by NTIA; CPUC; federal, state, and local jurisdictions where subject cultural resources are identified; and the State Historic Preservation Officer (SHPO). All recommended measures shall be implemented by CVIN.

Environmental Protection Measure 4.7-2: Implement the Archaeological Monitoring and Discovery Plan.

Before the start of construction under the Proposed Action and any ground disturbance, CVIN shall retain the services of a qualified archaeologist who meets the requirements of the Secretary of the Interior’s Standards for Archaeology and Historic Preservation and is familiar with local conditions to implement the Archaeological Monitoring and Discovery Plan (AMDP) which is included in the confidential Cultural Resources Inventory Report (AECOM 2011c).

The AMDP requires that a qualified archaeologist conduct monitoring for all excavation activity that occurs in areas of potential cultural sensitivity. The AMDP identifies specific locations where monitoring shall be conducted. The AMDP defines how archaeological monitoring will be conducted and the protocol to be followed in the event that significant resources are discovered during monitoring. Areas of potential paleontological resource sensitivity shall similarly be assessed in accordance with Environmental Protection Measure 4.7-6 which also describes the procedures to be followed in the event of discovery of paleontological resources.

The AMDP specifies that before construction or ground-disturbing activities associated with the Proposed Action begin, all construction personnel shall be alerted to the possibility of buried cultural resources anywhere along the project route. Should any cultural resources, such as structural features, unusual amounts of bone or shell, artifacts, human remains, or paleontological resources (see Environmental Protection Measure 4.7-6 below) be encountered during project construction activity, work shall be suspended within 100 feet of the excavation, until the qualified archaeologist has inspected and evaluated the discovery. NTIA and CPUC shall be notified immediately of the discovery. Once approved by NTIA and CPUC, work may proceed on other portions of the construction segment(s) while mitigation of impacts on archaeological resources are implemented. If a resource that is potentially eligible for listing in the NRHP or the CRHR is identified, the provisions of Environmental Protection Measure 4.7-1, “Implement Specific Measures for Full Avoidance of Adverse Effects on Each Cultural Resource Potentially Eligible for Listing in the National Register of Historic Places or the California Register of Historical Resources;” Environmental Protection Measure 4.7-3, “Strictly Follow Procedures for the Discovery of Unanticipated Cultural Resources, and If Discovered, Immediately Cease Construction Activities, Notify the National Telecommunications Information Administration and California Public Utilities Commission, and

Implement all Appropriate Avoidance Measures or Other Appropriate Mitigation in Consultation with the Lead Agencies and State Historic Preservation Officer” and Environmental Protection Measure 4.7-4, “Strictly Follow Procedures for the Discovery of Unanticipated Human Remains, and If Discovered, Immediately Cease Construction Activities, Notify the National Telecommunications Information Administration, California Public Utilities Commission, and the Affected County Coroner, and Implement All Appropriate Mitigation in Consultation with the Native American Heritage Commission, If the Remains are Determined to be Native American” shall be implemented.

The AMDP has been reviewed by NTIA and the California SHPO and the provisions included in the AMDP were referenced in the letter from SHPO as stipulations for SHPO’s concurrence in the finding of No Adverse Effect on historic properties (see Appendix H). CVIN shall adhere to and implement all recommendations included in the AMDP.

Environmental Protection Measure 4.7-3: Strictly Follow Procedures for the Discovery of Unanticipated Cultural Resources.

If any prehistoric or historic subsurface cultural resources are discovered during ground-disturbing activities, all work within 100 feet of the resources shall be halted and CVIN shall notify NTIA, CPUC, California State Lands Commission (CSLC) and the local or state agency with jurisdiction over the project and shall consult with a qualified archaeologist to assess the significance of the find. If the discovery is made on BLM land, the BLM Mother Lode Field Office shall be contacted. If any find is determined to be significant, representatives of CVIN, NTIA, CPUC, CSLC the local or state agency, and the archaeologist shall meet to determine the appropriate avoidance measures or other appropriate mitigation, with the ultimate determination to be made by NTIA, CPUC, CSLC and the local or state agency in consultation with the SHPO. All significant cultural materials recovered shall be subject to scientific analysis, professional museum curation, and a report prepared by an archaeologist according to current professional standards, as necessary.

In considering any suggested mitigation proposed by the archaeologist to mitigate impacts on historical resources or unique archaeological resources, NTIA, CPUC, CSLC, and the local or state agency shall determine whether avoidance is necessary and feasible in light of factors such as the nature of the find, design, costs, and other considerations. If avoidance is infeasible, other appropriate measures (e.g., data recovery) shall be implemented in consultation with the SHPO. Work may proceed on other parts of the construction segment(s) while mitigation for historical resources or unique archaeological resources is carried out.

If NTIA, CPUC, CSLC and the local or state agency, in consultation with the archaeologist, determine that a significant archaeological resource is present and that the resource could be adversely affected by the Proposed Action, NTIA, CPUC, CSLC and the local or state agency shall require CVIN to:

- redesign the project to avoid any adverse impact on the significant archaeological resource, if feasible; or

- implement the ADRP (unless the archaeologist determines that the archeological resource is of greater interpretive use than research significance and that interpretive use of the resource is feasible). Environmental Protection Measure 4.7-4: Strictly Follow Procedures for the Discovery of Unanticipated Human Remains.

If human remains are discovered during any phase of construction, work within 100 feet of the remains shall be suspended immediately and NTIA, CPUC, and the coroner for the county in which the remains are discovered shall be immediately notified. If the remains are determined by the County Coroner to be Native American, the Native American Heritage Commission (NAHC) shall be notified within 24 hours, and the guidelines of the NAHC shall be adhered to in the treatment and disposition of the remains. CVIN shall also retain a professional archaeologist with Native American burial experience to conduct a field investigation of the specific site and consult with the Most Likely Descendant, if any, identified by the NAHC. As necessary, the archaeologist may provide professional assistance to the Most Likely Descendant, including the excavation and removal of the human remains. CVIN shall implement approved mitigation, which shall be verified by CPUC before construction activities may resume at the site where the remains were discovered.

If human remains of Native American origin are discovered on federal land during ground-disturbing activities, compliance with the Native American Graves Protection and Repatriation Act is required. This law specifies the procedures that agencies must follow when burials of Native American origin are discovered on federal land (43 Code of Federal Regulations [CFR] Part 10). These regulations include the following provisions, which shall be implemented by CVIN and by the federal agency with direct jurisdiction over the land (such as BLM) if human remains are discovered during construction activities.

- Notify the applicable county coroner or the sheriff.

- Notify the responsible federal agency, in writing.

- Cease all activity in the area of the discovery and protect the human remains.

- Upon notification that the human remains have been discovered on federal land, the responsible federal agency shall:

- certify receipt of the notification,

- take steps to secure and protect the human remains,

- notify the Native American tribe or tribes likely to be culturally affiliated with the discovered human remains within one working day, and

- initiate consultation with the Native American tribe or tribes in accordance with regulations described in 43 CFR 10.5.

Environmental Protection Measure 4.7-5: Perform a Site-Specific Paleontological Resources Inventory and Assessment by Rock Unit and Conduct Monitoring in Paleontologically Sensitive Areas.

Before the start of earthmoving activities, CVIN shall retain the services of a qualified professional paleontologist (i.e., with at least a B.S. in Geology and an M.S. or higher in Paleontology, plus at least 2 years of field experience) who shall prepare and implement a paleontological resources inventory and assessment by rock unit. This report shall include the components described below.

- A report of any fossils observed during a reconnaissance-level field survey. (At CVIN’s discretion, rather than having a separate field reconnaissance visit solely to evaluate paleontological resources, CVIN may retain the qualified professional archaeologist surveying for cultural resources to simultaneously conduct the survey for paleontological resources. If this option is selected, the archaeologist shall provide the results of the reconnaissance survey to the paleontologist retained by CVIN for inclusion in the paleontological resources inventory and assessment by rock unit.)

- The results of a records search of appropriate paleontological databases (at a minimum, the database at the University of California, Berkeley Museum of Paleontology [UCMP]) to determine whether any previously recorded fossil localities are located within or immediately adjacent to the proposed infrastructure facilities.

- A complete listing of all geologic formations within the alignment of the proposed infrastructure.

- A determination as to whether the geologic formations are of high or low paleontological sensitivity, and a discussion supporting the reasons why the sensitivity determinations were made.

Before the start of earthmoving activities within any geologic formations determined to be paleontologically sensitive, the qualified professional paleontologist retained by CVIN shall train all construction personnel involved with earthmoving activities, including the site superintendent, regarding the possibility of encountering fossils, the appearance and types of fossils likely to be seen during construction, and proper notification procedures should fossils be encountered.

CVIN shall conduct full-time monitoring during earthmoving activities in the paleontologically sensitive rock formations identified during the field reconnaissance and documented in the report. (At CVIN’s discretion, rather than having both a paleontologist and an archaeologist present in locations that require monitoring, CVIN may have a qualified archaeologist monitor for both resources simultaneously.)

Environmental Protection Measure 4.7-6: Stop Work If Paleontological Resources are Encountered and Prepare and Implement a Recovery Plan.

If paleontological resources are discovered during earthmoving activities, the construction crew shall immediately cease work in the vicinity of the find and notify CVIN, NTIA, CPUC, BLM (if on BLM lands), and the planning department of the relevant county or city where the work is occurring. CVIN shall retain a qualified paleontologist to evaluate the resource and prepare a recovery plan in accordance with Society of Vertebrate Paleontology guidelines (1996). The recovery plan may include but is not limited to a site-specific field survey, construction monitoring, sampling and data recovery procedures, museum storage coordination for any specimen recovered, and a report of findings. Recommendations in the recovery plan that are determined by the lead agencies to be necessary and feasible shall be implemented before construction activities can resume at the site where the paleontological resources were discovered.

##### Aesthetic and Visual Resources

Environmental Protection Measure 4.8-1: Implement Environmental Protection Measure 4.7-1 for Attaching Conduit to Historic Bridges, and If Rerouting or Directional Drilling (Boring) Is Infeasible, Paint Conduits Attached to Historic Bridges and Nonhistoric Bridges Crossing Scenic Waterways the Same Color as the Bridge.

CVIN shall implement Environmental Protection Measure 4.7-1, “Implement Specific Measures for Full Avoidance of Adverse Effects on Each Cultural Resource Potentially Eligible for Listing in the National Register of Historic Places or the California Register of Historical Resources,” for attaching conduit to historic bridges. However, if rerouting or directional drilling (boring) is infeasible, or if the conduit would be attached to a nonhistoric bridge crossing any scenic waterway, the conduits shall be painted to match the color of the bridge.

##### Infrastructure

Environmental Protection Measure 4.10-1: Verify Utility Locations, Coordinate with Utility Providers, Prepare and Implement a Response Plan, and Conduct Worker Training with Respect to Accidental Utility Damage.

Before the start of construction, CVIN and its construction contractors shall implement the measures listed below.

- Notify appropriate agencies of any potential interruptions in service.

- Through field surveys and use of the Underground Service Alert services, verify the locations of buried utility lines in the area of construction. Clearly mark these locations on the construction specifications in advance of any drilling or trenching.

- Prepare and implement (when necessary) an accidental-damage response plan that includes the chain-of-command rules for notifying authorities, a list of the appropriate actions and responsibilities to ensure the safety of workers and the public, and training of workers on appropriate actions and responsibilities in response to accidental damage to utility lines.

##### Human Health and Safety

Environmental Protection Measure 4.12-1: Prepare and Implement a Spill Prevention Plan.

CVIN shall prepare a spill prevention plan to be implemented in each staging area. Construction contractors and workers shall receive written instructions and training on the plan, which is intended to reduce the potential risk of accidental spills in construction areas along the proposed route and at cabinet sites. The spill prevention plan for each staging area shall include the measures listed below.

- Requirements pertaining to periodic maintenance and refueling of equipment used for construction. The following specific requirements shall be included in the spill preparation plan:

- To reduce the potential for contamination by spills, no refueling, storage, servicing, or maintenance of equipment shall take place within 150 feet of drainages, sensitive waterways, or other sensitive environmental resources.

- No construction equipment shall be refueled or serviced without absorbent material or drip pans underneath to contain spilled fuel.

- Any fluids drained from the machinery during servicing shall be collected in leakproof containers and taken to an appropriate disposal or recycling facility.

- Under no circumstances shall contaminated soils be added to a spoils pile.

- All equipment wash-down activities shall be conducted at least 150 feet away from sensitive environmental resources.

- A hazardous substance control and emergency response plan addressing preparations for quick and safe cleanup of accidental spills. The plan shall prescribe procedures for handling hazardous materials to reduce the potential for a spill during construction and shall include an emergency response program. The plan shall identify areas where refueling and vehicle maintenance activities and storage of hazardous materials, if any, would be permitted.

- An environmental training and monitoring program to communicate environmental concerns and appropriate work practices, including spill prevention, emergency response measures, and applicable BMPs, to all construction and operations personnel. A monitoring program shall be implemented to ensure that the plans are followed during construction.

Environmental Protection Measure 4.12-2: Conduct Construction Soil Sampling and Testing If Soil Contamination Is Suspected and if Hazardous Materials are Encountered, Cease all Construction in the Area until the Material is Properly Characterized and Appropriate Measures are Implemented.

Soil sampling shall be conducted in the project footprint, as needed, before construction begins. Soil information shall be provided to construction crews to inform them about soil conditions and potential hazards. If hazardous substances are unexpectedly encountered during trenching, grading, or excavating work, work shall be stopped until the material is properly characterized and appropriate measures are taken to protect human health and the environment. If excavation of hazardous materials is required, the materials shall be handled, transported, and disposed of in accordance with federal, state, and local regulations.

Environmental Protection Measure 4.12-3: Conduct Groundwater Sampling and Testing If Suspected Contaminated Groundwater Is Encountered during Construction and if Contamination is Encountered Collect, Contain, and Dispose of it Properly.

If suspected contaminated groundwater is encountered in the construction areas, samples shall be collected and submitted for analysis of petroleum hydrocarbons, metals, volatile organic compounds, and semivolatile organic compounds. If necessary, groundwater shall be collected during construction, contained, and disposed of in accordance with all applicable regulations.

Environmental Protection Measure 4.12-4: Prepare and Implement a Health and Safety Plan.

CVIN shall prepare and implement a health and safety plan that addresses emergency medical services to be provided in case of an emergency. The plan shall list procedures, specific emergency response, and evacuation measures to be followed during emergencies. This manual shall be distributed by the construction contractors to all workers before the start of construction and during operation of the Proposed Action. Maps describing locations of towers, poles, and access roads shall be provided to emergency personnel to ensure proper emergency response to all parts of the proposed route and cabinet sites.

Environmental Protection Measure 4.12-5: Prepare and Implement a Fire Risk Management Plan.

CVIN shall prepare and implement a fire risk management plan that addresses fire-suppression equipment and procedures to be used during construction and training of construction and maintenance crews. Additionally, fire suppression equipment and materials shall be kept adjacent to all work areas and in staging areas, and shall be clearly marked. Detailed information for responding to fires shall be provided in the Proposed Action’s fire risk management plan. Information contained in the plan and locations of fire suppression materials and equipment shall be included as part of the employee environmental training addressed in Environmental Protection Measure 4.12-1, “Prepare and Implement a Spill Prevention Plan.”

Environmental Protection Measure 4.12-6: Prepare and Implement Traffic Control Plans.

CVIN’s construction contractor(s), in coordination with local traffic regulatory agencies, the California Highway Patrol, Caltrans, and local emergency services, shall prepare and implement traffic control plans for project construction to reduce the effects of construction on roadway and highway systems throughout the construction period. The traffic control plans shall contain elements on detour routing, flagging, and measures to ensure emergency access through the construction area and to adjacent properties. Proposed lane closures during the a.m. and p.m. peak hours shall be minimized. Lane closures shall be limited to the immediate vicinity of the open trench. Pedestrian and bicycle access shall be rerouted around project sites at all times. During construction, the construction sites shall be secured to prevent pedestrian and bicyclists from entering the sites.

The traffic control plans shall be submitted for approval to the appropriate traffic regulatory agency, NTIA, and CPUC at least 60 days before construction begins, for each road encroachment where trenching or other work within roadways would be conducted. Traffic control plans shall be prepared and implemented for construction activities that would directly or indirectly disturb the local traffic flow at each roadway encroachment location.

Environmental Protection Measure 4.12-7: Notify Property Owners Concerning Blocked Driveways, Limit Hours of Disruption of Driveways, or Compensate Property Owners.

Access to all private roadways and driveways shall not be disrupted for more than 4 hours at a time. CVIN shall notify property owners of the roadways and driveways proposed to be blocked by construction activities at least 2 weeks before and again 48 hours before the access disruption. CVIN shall also provide alternate access or other compensation to private residences that are disrupted for more than 4 hours at a time, in accordance with an agreement developed between CVIN and the property owner.

Environmental Protection Measure 4.12-8: Reduce Potential Roadway Damage Resulting from Construction.

To reduce potential roadway damage effects, CVIN shall implement the following measures:

- record views of the roadway and access roads on videotape before and after construction to document the existing and restored roadways;

- make temporary repairs from roadway damage as necessary during construction;

- repair any damaged roadway to its original condition immediately after construction has been completed;

- coordinate with Caltrans or the local traffic regulatory agency, as applicable, to determine appropriate routes for truck travel before beginning construction; and

- coordinate with the local traffic regulatory agency regarding planned improvements near the facility to limit interference with the implementation of roadway improvements or trenching in nearly completed facilities before beginning construction.